# Documentation and Resources Guide

Human-Centric EnMS Project

*Comprehensive Documentation for Software, Hardware, and Reusability*

# 📋 Table of Contents

[Overview](#overview)

[Repository Access](#repository-access)

[Software Documentation](#software-documentation)

[Hardware Documentation](#hardware-documentation)

[Bill of Materials (BOM)](#bill-of-materials-bom)

[Code Resources](#code-resources)

[Technical Manuals](#technical-manuals)

[Reusability Guidelines](#reusability-guidelines)

# Overview

This document serves as a comprehensive index of all technical documentation, code repositories, hardware specifications, and resources for the Human-Centric EnMS (Energy Management System) project. It ensures complete transparency and reproducibility for all stakeholders, developers, and auditors.

The Human-Centric EnMS platform is an IoT-based, real-time energy monitoring and Digital Product Passport (DPP) generation system designed for additive manufacturing environments. All components are documented with sufficient detail to enable independent verification, modification, and deployment.

# Repository Access

## Official Repositories

Primary Repository (GitHub):

URL: https://gitlab.com/lauds/enms-project

Purpose: Main development repository with full version history

Access: Public (Open Source)

Demo:

URL: https://lauds-demo.intel50001.com

Purpose: Live demonstration environment with simulated data

## Cloning the Repository

# Clone from Gitlab

git clone https://gitlab.com/lauds/enms-project.git

cd enms-project

# Software Documentation

## Core Documentation Files

All documentation is located in the root directory of the repository in Markdown format:

| Document | File | Purpose |
| --- | --- | --- |
| User Guide | ui_guide.md | Comprehensive guide to all software features, user profiles, and artistic visualizations |
| README | README.md | Quick start guide, installation instructions, and architecture overview |
| API Documentation | DPP_API_Documentation.md | Complete REST API reference for Digital Product Passport endpoints |
| Technical Details | ENMS_Technical_Details.md | System architecture, design decisions, and technical implementation details |
| Analysis Deep Dive | ANALYSIS_DEEP_DIVE.md | Machine learning models, feature engineering, and statistical analysis |
| Interactive Analysis | INTERACTIVE_ANALYSIS_GUIDE.md | Step-by-step guide for the interactive analysis feature |
| Hardware Guide | Custom Hardware.md | ESP32 sensor hub specifications, wiring diagrams, and integration guide |

## Documentation Coverage

The documentation provides:

✅ Installation & Deployment: Complete Docker Compose setup with zero-touch deployment

✅ User Workflows: Three distinct user profiles (Technical, Staff, DPP) with role-specific features

✅ API Specifications: All REST endpoints with request/response schemas

✅ Database Schema: PostgreSQL + TimescaleDB table structures and relationships

✅ ML Model Training: Data preprocessing, feature engineering, model selection, and evaluation

✅ Frontend Architecture: HTML/CSS/JavaScript structure and component interactions

✅ Backend Workflows: Node-RED flows, MQTT topics, and data pipelines

✅ Custom Hardware: Sensor specifications, firmware, and MQTT integration

# Hardware Documentation

## Custom Hardware Components

Location: Custom Hardware.md

The system integrates custom IoT sensor hubs for enhanced monitoring capabilities beyond standard 3D printer APIs.

### ESP32 Sensor Hub

Purpose: Real-time environmental and operational monitoring

Main Components:

1. ESP32 Microcontroller

Model: ESPDuino-32 ESP32 WiFi and Bluetooth Development Board

Purpose: WiFi-enabled dual-core processor for sensor data collection and MQTT publishing

Quick Spec:

Input Voltage 7-12V (recommended)

IO Pins are 3.3V and not 5V

Arduino UNO Architecture

Soft Serial or Hardware Serial Connection

Integrated ESP-Wroom-32 Wifi Module

Firmware: Custom C++ code for sensor polling and MQTT communication

2. MPU6050 (6-Axis IMU)

Purpose: 3-axis accelerometer + 3-axis gyroscope for vibration analysis

Use Case: Operational state detection (Printing/Idle/Off), predictive maintenance

Interface: I2C

Output: Acceleration (g), Angular velocity (°/s)

3. MAX6675 Thermocouple Amplifier

Purpose: High-precision temperature measurement via K-type thermocouple

Temperature Range: 0°C to 1024°C

Resolution: 0.25°C

Interface: SPI

Use Case: Hotend, bed, or enclosure temperature monitoring

4. DHT22 Sensor

Purpose: Ambient temperature and humidity monitoring

Temperature Range: -40°C to 80°C (±0.5°C)

Humidity Range: 0-100% RH (±2%)

Interface: Single-wire digital

Use Case: Environmental condition tracking for print quality correlation

Connectivity:

Protocol: MQTT over WiFi

MQTT Topics:

enms/sensor/<device_id>/accel - Accelerometer data

enms/sensor/<device_id>/gyro - Gyroscope data

enms/sensor/<device_id>/ambient - DHT22 temperature/humidity

enms/sensor/<device_id>/thermocouple - MAX6675 temperature

Data Flow:

ESP32 Sensors → WiFi → MQTT Broker (Mosquitto) → Node-RED → PostgreSQL (TimescaleDB)

Visual Reference:

Sensor Hub Photo: docs/Sensor Hub.png

MAX6675 Module: docs/MAX6675.png

# Bill of Materials (BOM)

## ESP32 Sensor Hub - Hardware Components

| Quantity | Component | Model/Type | Purpose |
| --- | --- | --- | --- |
| 1 | Microcontroller | ESP32-WROOM-32 Development Board | Main processor with WiFi |
| 1 | Accelerometer/Gyroscope | MPU6050 6-Axis IMU Module | Vibration and motion detection |
| 1 | Thermocouple Amplifier | MAX6675 K-Type Module | High-temperature measurement |
| 1 | K-Type Thermocouple | Standard K-Type Probe | Temperature sensor probe |
| 1 | Temperature/Humidity Sensor | DHT22 (AM2302) | Ambient environmental monitoring |
| 1 | Breadboard or PCB | Standard 830-point breadboard or custom PCB | Component mounting |
| - | Jumper Wires | Male-to-Male and Male-to-Female | Connections |
| 1 | Power Supply | 5V 2A USB Power Adapter | ESP32 power source |
| 1 | USB Cable | Micro-USB or USB-C (depending on ESP32 board) | Programming and power |

Notes:

PCB design files can be created for production runs (currently breadboard prototype)

## Software Components (Free/Open Source)

| Component | License | Purpose |
| --- | --- | --- |
| Docker & Docker Compose | Apache 2.0 | Container orchestration |
| PostgreSQL + TimescaleDB | PostgreSQL License | Time-series database |
| Node-RED | Apache 2.0 | Data flow automation |
| Grafana | AGPL v3 | Visualization dashboards |
| Mosquitto MQTT Broker | EPL/EDL | Message broker |
| Python Flask | BSD-3-Clause | REST API service |
| Nginx | 2-clause BSD | Web server and reverse proxy |
| WeasyPrint | BSD-3-Clause | PDF generation |

## Optional Hardware

| Component | Purpose |
| --- | --- |
| Raspberry Pi 4 (4GB) | Bridge for legacy printers via OctoPrint |
| Shelly Plug S | Smart plug for power monitoring |

# Code Resources

## Repository Structure

enms-project/

├── backend/

│   ├── db_init/              # PostgreSQL initialization scripts

│   │   ├── 01_schema.sql     # Database schema (tables, hypertables)

│   │   ├── 02_data.sql       # Sample data and initial records

│   │   ├── 03_functions.sql  # Custom SQL functions and triggers

│   │   └── 04_auth_schema.sql # Authentication tables

│   ├── train_model.py        # Model training pipeline

│   ├── prediction_worker_mqtt.py # Real-time prediction worker

│   ├── export_training_data.py # Training data export

│   └── models/               # ML model artifacts directory

│

├── frontend/

│   ├── index.html            # Main UI entry point

│   ├── dpp_page.html         # Digital Product Passport carousel

│   ├── device_management.html # Printer fleet configuration

│   ├── auth.html             # Login/registration page

│   ├── analysis/             # Interactive analysis feature

│   └── components/           # Reusable UI components

│

├── grafana/

│   ├── provisioning/

│   │   ├── datasources/      # TimescaleDB datasource config

│   │   └── dashboards/       # Dashboard provisioning

│   └── dashboards/           # JSON dashboard definitions

│       ├── fleet-operations.json

│       ├── Machine-Performance-Comparison.json

│       ├── Industrial-Hybrid-Edge-System.json

│       ├── Sensor-Data-Explorer.json

│       └── esp32.json

│

├── node-red/

│   ├── flows.json            # Node-RED flow definitions

│   ├── settings.js           # Node-RED configuration

│   └── nodes/                # Custom Node-RED nodes

│

├── python-api/

│   ├── app.py                # Flask application entry point

│   ├── dpp_simulator.py      # DPP data generation logic

│   ├── pdf_service.py        # PDF report generation

│   ├── gcode_analyzer.py     # G-code metadata extraction

│   └── templates/            # HTML templates for PDF reports

│

├── nginx/

│   └── conf.d/

│       └── default.conf      # Nginx routing configuration

│

├── artistic-resources/

│   └── plants/               # Energy plant visualization images

│       ├── potato/           # 21 growth stages

│       ├── corn/             # 8 growth stages

│       ├── corn_2/           # 12 growth stages

│       └── corn_3/           # 7 growth stages

│

└── docker-compose.yml        # Complete service orchestration

## Key Source Files

Database Schema:

backend/db_init/01_schema.sql - Complete table definitions, indexes, and hypertables

Backend API:

python-api/app.py - REST API endpoints

python-api/dpp_simulator.py - Energy plant stage calculation, DPP data aggregation

python-api/pdf_service.py - WeasyPrint PDF generation with plant images

Frontend:

frontend/index.html - Profile selection and main dashboard

frontend/dpp_page.html - Interactive 3D carousel

frontend/analysis/analysis_page.js - Statistical analysis and ML feature importance

Data Pipeline:

node-red/flows.json - Complete data ingestion, transformation, and routing logic

Machine Learning:

backend/train_model.py - Model training pipeline (XGBoost, RandomForest, LightGBM)

backend/prediction_worker_mqtt.py - Real-time energy prediction worker

# Technical Manuals

## Installation Manual

### Prerequisites

Docker Engine: Version 20.10 or higher

Docker Compose: Version 2.0 or higher

System Resources:

RAM: 4GB minimum (8GB recommended)

Storage: 20GB free disk space

OS: Linux (Ubuntu 20.04+), macOS, or Windows with WSL2

### Installation Steps

1. Clone the Repository

git clone https://gitlab.com/lauds/enms-project.git

cd enms-project

2. Configure Environment Variables

Create the environment file from the provided template:

cp .env.example .env

Edit .env with your preferred text editor and update the following critical variables:

MQTT_PASSWORD - Set a secure password for MQTT broker

NODE_RED_CREDENTIAL_SECRET - Generate a random secret key

POSTGRES_PASSWORD - Set database password (default: enms_pass)

3. Build and Start All Services

docker compose up --build -d

This command will:

Download all required Docker images

Build custom images for Node-RED and Python services

Initialize PostgreSQL database with schema and sample data

Configure MQTT broker with authentication

Start all services in detached mode

4. Verify Deployment

docker compose ps

All services should show status as "Up" or "running".

5. Access the Application

| Service | URL | Default Credentials |
| --- | --- | --- |
| Main UI | http://localhost:8090 | demo / demo123 |
| Node-RED | http://localhost:1882 | (see .env) |
| Grafana | http://localhost:3000 | admin / (see .env) |
| PostgreSQL | localhost:5432 | enms_user / (see .env) |

Estimated Setup Time: 10-15 minutes (depending on internet speed)

### Post-Installation Configuration

Add Devices: Navigate to Device Management page and add your 3D printers

Import Grafana Dashboards: Dashboards are auto-provisioned on first startup

Configure SimplyPrint Integration: Add your SimplyPrint API credentials in Node-RED settings

## User Manual

### System Overview

The Human-Centric EnMS platform provides three distinct user interfaces tailored to different roles:

1. Technical Profile

Purpose: Detailed operational monitoring and analysis

Features:

Real-time printer status dashboard

Energy consumption analytics

Machine performance comparison

Sensor data visualization

Interactive analysis tools

2. Staff Profile

Purpose: Simplified monitoring for operations staff

Features:

Fleet status overview

Active job monitoring

Energy plant visualizations (artistic representation)

Simplified metrics

3. DPP (Digital Product Passport) Profile

Purpose: Product-focused documentation and traceability

Features:

3D carousel of completed jobs

PDF report generation

G-code preview and analysis

Energy footprint per product

Environmental impact visualization

### Key Features

Energy Plant Visualizations

The system uses an artistic metaphor where energy consumption is visualized as a growing plant:

potato - 21 growth stages (high-energy products)

corn - 8 growth stages (standard products)

corn_2 - 12 growth stages (medium products)

corn_3 - 7 growth stages (small products)

Each stage represents a threshold of energy consumption, making sustainability tangible and engaging.

PDF Report Generation

Users can generate comprehensive reports including:

Job metadata (duration, material, settings)

Energy consumption breakdown

3D G-code preview rendering

Environmental impact metrics

Growth stage visualization

Interactive Analysis

Advanced statistical analysis tools:

Feature importance visualization

Correlation matrices

Energy prediction models

Performance trend analysis

## Developer Manual

### System Architecture

The Human-Centric EnMS follows a microservices architecture with the following components:

Data Flow:

3D Printers → SimplyPrint API → Node-RED → PostgreSQL (TimescaleDB)

↓

ESP32 Sensors → MQTT → Node-RED → PostgreSQL

↓

Python API ← Frontend (Web Browser)

↓

Grafana

### API Specifications

Base URL: http://localhost:8090/api

Key Endpoints:

1. GET /dpp_summary

Returns list of all completed print jobs

Response: Array of job objects with metadata

2. GET /dpp_report_data/:job_id

Returns detailed data for specific job

Includes energy metrics, plant visualization stage, G-code preview

3. POST /generate_pdf

Generates PDF report for specified job

Returns PDF file for download

4. GET /devices

Returns list of all registered devices

Includes device status and configuration

5. POST /devices

Registers new device

Request body: device_id, model, friendly_name, etc.

Authentication: Session-based authentication with username/password

### Database Schema

Core Tables:

1. devices - Printer fleet registry

device_id (PK) - Unique printer identifier

device_model - Printer model

friendly_name - Display name

simplyprint_id - SimplyPrint integration ID

location - Physical location

2. print_jobs - Completed print job records

job_id (PK) - Auto-incrementing job ID

device_id (FK) - References devices

start_time, end_time - Job duration

energy_kwh - Total energy consumed

material_used_grams - Filament weight

3. energy_readings (Hypertable) - Time-series energy data

time - Timestamp (indexed by TimescaleDB)

device_id (FK) - References devices

power_w - Instantaneous power reading

cumulative_kwh - Running total

4. sensor_data (Hypertable) - ESP32 sensor readings

time - Timestamp

device_id (FK)

accel_x, accel_y, accel_z - Accelerometer data

gyro_x, gyro_y, gyro_z - Gyroscope data

temperature, humidity - Environmental sensors

### MQTT Topics

Published by ESP32 Sensors:

enms/sensor/<device_id>/accel - Accelerometer data (JSON)

enms/sensor/<device_id>/gyro - Gyroscope data (JSON)

enms/sensor/<device_id>/ambient - DHT22 temperature/humidity

enms/sensor/<device_id>/thermocouple - MAX6675 temperature

Message Format Example:

{

"device_id": "ender3_01",

"timestamp": "2025-12-09T10:30:00Z",

"accel_x": 0.12,

"accel_y": -0.05,

"accel_z": 9.81

}

### Machine Learning Models

Training Pipeline:

1. Data extraction from TimescaleDB

2. Feature engineering (rolling averages, variance, time-based features)

3. Model training with cross-validation

4. Model selection (XGBoost, RandomForest, LightGBM)

5. Serialization with joblib

Features Used:

Print duration

Layer height

Infill percentage

Print temperature

Print speed

Material type

Bed temperature

Target Variable: energy_kwh (total energy consumption)

## Hardware Integration Manual

### ESP32 Sensor Hub Setup

Required Hardware:

ESP32 development board (ESPDuino-32 recommended)

MPU6050 6-axis IMU module

MAX6675 thermocouple amplifier

K-type thermocouple probe

DHT22 temperature/humidity sensor

Breadboard and jumper wires

5V USB power adapter

Wiring Diagram:

MPU6050 (I2C):

VCC → ESP32 3.3V

GND → ESP32 GND

SCL → ESP32 GPIO 22

SDA → ESP32 GPIO 21

MAX6675 (SPI):

VCC → ESP32 3.3V

GND → ESP32 GND

SCK → ESP32 GPIO 18

CS → ESP32 GPIO 5

SO → ESP32 GPIO 19

DHT22:

VCC → ESP32 3.3V

GND → ESP32 GND

DATA → ESP32 GPIO 4

Firmware Configuration:

Edit the Arduino sketch with your WiFi and MQTT credentials:

const char* ssid = "Your_WiFi_SSID";

const char* password = "Your_WiFi_Password";

const char* mqtt_server = "your_mqtt_broker_ip";

const char* mqtt_user = "enms_mqtt_user";

const char* mqtt_pass = "your_mqtt_password";

Upload Firmware:

1. Install Arduino IDE

2. Add ESP32 board support (ESP32 Board Manager URL)

3. Install libraries: MPU6050, MAX6675, DHT, PubSubClient

4. Compile and upload sketch to ESP32

5. Open Serial Monitor to verify connection

Troubleshooting:

WiFi not connecting: Verify SSID/password, check 2.4GHz band

MQTT publish fails: Confirm broker IP, check firewall rules

Sensor not detected: Verify wiring, check I2C address with scanner

MAX6675 reads 0: Ensure thermocouple polarity is correct

# Software Code Documentation

## Docker Compose Configuration

The complete system orchestration is defined in docker-compose.yml:

services:

postgres:

image: timescale/timescaledb:latest-pg16

container_name: enms_postgres

restart: unless-stopped

environment:

- POSTGRES_DB=${POSTGRES_DB}

- POSTGRES_USER=${POSTGRES_USER}

- POSTGRES_PASSWORD=${POSTGRES_PASSWORD}

volumes:

- postgres_data:/var/lib/postgresql/data

- ./backend/db_init:/docker-entrypoint-initdb.d

ports:

- "${POSTGRES_PORT}:5432"

healthcheck:

test: ["CMD-SHELL", "pg_isready -U ${POSTGRES_USER} -d ${POSTGRES_DB}"]

interval: 10s

timeout: 5s

retries: 5

mosquitto:

image: eclipse-mosquitto:2

container_name: enms_mosquitto

restart: unless-stopped

ports:

- "1884:1883"

volumes:

- mosquitto_config:/mosquitto/config

- mosquitto_data:/mosquitto/data

nodered:

build:

context: .

dockerfile: ./node-red/Dockerfile

container_name: enms_nodered

restart: unless-stopped

ports:

- "1882:1880"

volumes:

- ./node-red:/data

- ./backend/models:/models

environment:

- POSTGRES_DB=${POSTGRES_DB}

- POSTGRES_USER=${POSTGRES_USER}

- POSTGRES_PASSWORD=${POSTGRES_PASSWORD}

- MQTT_BROKER_HOST=${MQTT_BROKER_HOST}

- MQTT_USERNAME=${MQTT_USERNAME}

- MQTT_PASSWORD=${MQTT_PASSWORD}

depends_on:

- postgres

- mosquitto

python-api:

build:

context: ./python-api

container_name: enms_python_api

restart: unless-stopped

environment:

- DATABASE_URL=postgresql://${POSTGRES_USER}:${POSTGRES_PASSWORD}@postgres:5432/${POSTGRES_DB}

volumes:

- generated_pdfs:/app/generated_pdfs

- ./artistic-resources:/app/artistic-resources

depends_on:

- postgres

grafana:

image: grafana/grafana:latest

container_name: enms_grafana

restart: unless-stopped

ports:

- "3000:3000"

environment:

- GF_SECURITY_ADMIN_PASSWORD=${GRAFANA_ADMIN_PASSWORD}

volumes:

- grafana_data:/var/lib/grafana

- ./grafana/provisioning:/etc/grafana/provisioning

- ./grafana/dashboards:/var/lib/grafana/dashboards

depends_on:

- postgres

nginx:

image: nginx:alpine

container_name: enms_nginx

restart: unless-stopped

ports:

- "8090:80"

volumes:

- ./frontend:/usr/share/nginx/html

- ./nginx/conf.d:/etc/nginx/conf.d

- generated_pdfs:/usr/share/nginx/html/generated_pdfs

depends_on:

- python-api

## Database Schema (PostgreSQL + TimescaleDB)

Key table definitions from backend/db_init/01_schema.sql:

-- Devices table: Printer fleet registry

CREATE TABLE public.devices (

device_id text NOT NULL PRIMARY KEY,

device_model text NOT NULL,

friendly_name text,

location text,

simplyprint_id text,

api_key text,

bed_width integer,

bed_depth integer,

last_seen timestamp with time zone

);

-- Print jobs table

CREATE TABLE public.print_jobs (

job_id integer NOT NULL PRIMARY KEY,

device_id text NOT NULL REFERENCES public.devices(device_id),

simplyprint_job_id text,

start_time timestamp with time zone,

end_time timestamp with time zone,

duration_seconds integer,

filename text,

filament_used_grams real,

energy_kwh real,

cost_estimate real,

status text,

thumbnail_url text

);

-- Energy readings (TimescaleDB hypertable)

CREATE TABLE public.energy_readings (

time timestamp with time zone NOT NULL,

device_id text NOT NULL REFERENCES public.devices(device_id),

power_w real,

cumulative_kwh real,

voltage real,

current real

);

-- Convert to hypertable for time-series optimization

SELECT create_hypertable('public.energy_readings', 'time', if_not_exists => TRUE);

-- Sensor data (TimescaleDB hypertable)

CREATE TABLE public.sensor_data (

time timestamp with time zone NOT NULL,

device_id text NOT NULL REFERENCES public.devices(device_id),

sensor_type text,

accel_x real,

accel_y real,

accel_z real,

gyro_x real,

gyro_y real,

gyro_z real,

temperature real,

humidity real,

thermocouple_temp real

);

SELECT create_hypertable('public.sensor_data', 'time', if_not_exists => TRUE);

-- Create indexes for performance

CREATE INDEX idx_energy_device_time ON public.energy_readings (device_id, time DESC);

CREATE INDEX idx_sensor_device_time ON public.sensor_data (device_id, time DESC);

CREATE INDEX idx_jobs_device ON public.print_jobs (device_id);

CREATE INDEX idx_jobs_time ON public.print_jobs (start_time DESC);

## Python Flask API

Core API implementation from python-api/app.py:

from flask import Flask, jsonify, request, send_file

import psycopg2

from psycopg2.extras import RealDictCursor

import os

app = Flask(__name__)

# Database connection

def get_db_connection():

conn = psycopg2.connect(os.environ['DATABASE_URL'])

return conn

# Get all completed jobs for DPP carousel

@app.route('/api/dpp_summary', methods=['GET'])

def dpp_summary():

conn = get_db_connection()

cursor = conn.cursor(cursor_factory=RealDictCursor)

query = """

SELECT

job_id, device_id, filename, start_time, end_time,

duration_seconds, filament_used_grams, energy_kwh,

thumbnail_url, status

FROM print_jobs

WHERE status = 'completed'

ORDER BY end_time DESC

LIMIT 50

"""

cursor.execute(query)

jobs = cursor.fetchall()

cursor.close()

conn.close()

return jsonify(jobs)

# Get detailed report data for specific job

@app.route('/api/dpp_report_data/<int:job_id>', methods=['GET'])

def dpp_report_data(job_id):

conn = get_db_connection()

cursor = conn.cursor(cursor_factory=RealDictCursor)

# Get job details

cursor.execute("SELECT * FROM print_jobs WHERE job_id = %s", (job_id,))

job = cursor.fetchone()

# Get energy data for job duration

cursor.execute("""

SELECT time, power_w, cumulative_kwh

FROM energy_readings

WHERE device_id = %s

AND time BETWEEN %s AND %s

ORDER BY time

""", (job['device_id'], job['start_time'], job['end_time']))

energy_data = cursor.fetchall()

# Calculate plant visualization stage

energy_kwh = job['energy_kwh'] or 0

plant_type, stage = calculate_plant_stage(energy_kwh)

cursor.close()

conn.close()

return jsonify({

'job': job,

'energy_data': energy_data,

'plant_type': plant_type,

'plant_stage': stage

})

# Plant stage calculation

def calculate_plant_stage(energy_kwh):

# Potato: 21 stages (high energy)

if energy_kwh > 5.0:

stage = min(int(energy_kwh / 0.5), 21)

return 'potato', stage

# Corn: 8 stages (standard)

elif energy_kwh > 2.0:

stage = min(int((energy_kwh - 2.0) / 0.4), 8)

return 'corn', stage

# Corn_2: 12 stages (medium)

elif energy_kwh > 0.5:

stage = min(int((energy_kwh - 0.5) / 0.15), 12)

return 'corn_2', stage

# Corn_3: 7 stages (small)

else:

stage = min(int(energy_kwh / 0.1), 7)

return 'corn_3', stage

if __name__ == '__main__':

app.run(host='0.0.0.0', port=5000)

## ESP32 Sensor Hub Firmware

Arduino sketch for ESP32 sensor hub:

#include <WiFi.h>

#include <PubSubClient.h>

#include <Wire.h>

#include <MPU6050.h>

#include <MAX6675.h>

#include <DHT.h>

// WiFi credentials

const char* ssid = "Your_WiFi_SSID";

const char* password = "Your_WiFi_Password";

// MQTT Configuration

const char* mqtt_server = "your_mqtt_broker_ip";

const int mqtt_port = 1884;

const char* mqtt_user = "enms_mqtt_user";

const char* mqtt_pass = "your_mqtt_password";

// Device configuration

const char* device_id = "ender3_01";

// Sensor objects

MPU6050 mpu;

MAX6675 thermocouple(18, 5, 19); // SCK, CS, SO pins

DHT dht(4, DHT22);

WiFiClient espClient;

PubSubClient client(espClient);

void setup() {

Serial.begin(115200);

Wire.begin(21, 22); // SDA, SCL

// Initialize sensors

mpu.initialize();

dht.begin();

// Connect to WiFi

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("WiFi connected");

// Configure MQTT

client.setServer(mqtt_server, mqtt_port);

}

void loop() {

if (!client.connected()) {

reconnect();

}

client.loop();

// Read sensors every 5 seconds

static unsigned long last_read = 0;

if (millis() - last_read > 5000) {

publishSensorData();

last_read = millis();

}

}

void reconnect() {

while (!client.connected()) {

if (client.connect(device_id, mqtt_user, mqtt_pass)) {

Serial.println("MQTT connected");

} else {

delay(5000);

}

}

}

void publishSensorData() {

// Read MPU6050

int16_t ax, ay, az, gx, gy, gz;

mpu.getMotion6(&ax, &ay, &az, &gx, &gy, &gz);

// Publish accelerometer

char topic[100];

char payload[200];

snprintf(topic, 100, "enms/sensor/%s/accel", device_id);

snprintf(payload, 200, "{\"x\":%.2f,\"y\":%.2f,\"z\":%.2f}",

ax/16384.0, ay/16384.0, az/16384.0);

client.publish(topic, payload);

// Publish gyroscope

snprintf(topic, 100, "enms/sensor/%s/gyro", device_id);

snprintf(payload, 200, "{\"x\":%.2f,\"y\":%.2f,\"z\":%.2f}",

gx/131.0, gy/131.0, gz/131.0);

client.publish(topic, payload);

// Read DHT22

float temp = dht.readTemperature();

float humidity = dht.readHumidity();

snprintf(topic, 100, "enms/sensor/%s/ambient", device_id);

snprintf(payload, 200, "{\"temperature\":%.1f,\"humidity\":%.1f}",

temp, humidity);

client.publish(topic, payload);

// Read MAX6675

float thermocouple_temp = thermocouple.readCelsius();

snprintf(topic, 100, "enms/sensor/%s/thermocouple", device_id);

snprintf(payload, 200, "{\"temperature\":%.2f}", thermocouple_temp);

client.publish(topic, payload);

}

# Reusability Guidelines

## Customization Points

Easy Modifications:

✅ Adding new plant types (artistic visualizations)

✅ Configuring energy thresholds for plant stages

✅ Customizing Grafana dashboards

✅ Modifying PDF report templates

✅ Adding new printer models to device management

Intermediate Modifications:

⚙️ Creating custom Node-RED flows for different data sources

⚙️ Adding new sensor types to ESP32 hub

⚙️ Extending REST API with new endpoints

⚙️ Implementing additional ML models

## Deployment Scenarios

Scenario 1: Small Workshop (1-5 printers)

Deploy on single server or workstation

Use default Docker Compose configuration

No clustering required

Estimated hardware: 8GB RAM, 4-core CPU

Scenario 2: Medium Facility (10-50 printers)

Deploy on dedicated server

Scale Node-RED workers

Add Grafana read replicas

Estimated hardware: 16GB RAM, 8-core CPU

Scenario 3: Enterprise (50+ printers)

Multi-node deployment

PostgreSQL clustering (TimescaleDB multi-node)

Load-balanced Nginx

Distributed MQTT brokers

Kubernetes orchestration recommended

## Extensibility Examples

1. New Printer Integrations: Custom printer types can be added through device management

2. Custom Sensors: ESP32 firmware can be extended with additional I2C/SPI devices

3. Alternative Databases: Node-RED flows can be modified to support InfluxDB, MongoDB, etc.

4. Custom Visualizations: 3D rendering can be extended for alternative file formats (STL, OBJ)

## Code Modification Examples

Adding a New Plant Type:

1. Add images to artistic-resources/plants/new_plant/ (sequential stages: 01.png, 02.png, etc.)

2. Update python-api/dpp_simulator.py:

def calculate_plant_stage(energy_kwh):

if energy_kwh > 10.0:  # New threshold

stage = min(int(energy_kwh / 0.8), 15)

return 'new_plant', stage

# ... existing logic

3. Rebuild Python API container

Adding a New API Endpoint:

@app.route('/api/custom_metric/<device_id>', methods=['GET'])

def custom_metric(device_id):

# Your custom logic

return jsonify({'result': 'data'})

Adding a New Sensor Type:

// In ESP32 firmware

#include <NewSensor.h>

NewSensor sensor(pin);

void publishNewSensor() {

float value = sensor.read();

snprintf(topic, 100, "enms/sensor/%s/new_sensor", device_id);

snprintf(payload, 200, "{\"value\":%.2f}", value);

client.publish(topic, payload);

}

# Verification and Testing

## Functional Tests

Location: backend/ (various test scripts)

Run Tests:

# Database schema verification

docker exec -it enms_postgres psql -U enms_user -d enms_db -f /docker-entrypoint-initdb.d/01_schema.sql

# ML model validation

python backend/train_model.py

## Data Integrity

Database Migrations: All schema changes are versioned in backend/db_init/ with sequential numbering

Backup/Restore:

# Backup database

docker exec enms_postgres pg_dump -U enms_user enms_db > backup.sql

# Restore database

docker exec -i enms_postgres psql -U enms_user enms_db < backup.sql

# Support and Contribution

## Getting Help

Documentation Issues: Open an issue on GitHub with label documentation

Bug Reports: Use issue template with reproduction steps

Feature Requests: Submit enhancement proposals via GitHub Discussions

## Contributing

Pull Request Process:

1. Fork the repository

2. Create feature branch (git checkout -b feature/amazing-feature)

3. Commit changes with descriptive messages

4. Push to branch

5. Open Pull Request with detailed description

Code Standards:

Python: PEP 8 compliance

JavaScript: ES6+ with consistent formatting

SQL: Uppercase keywords, consistent indentation

Markdown: Follow existing documentation style

# Appendix: Quick Reference

## Essential Commands

# Start all services

docker compose up -d

# View logs

docker compose logs -f [service_name]

# Stop all services

docker compose down

# Rebuild after code changes

docker compose up --build -d

# Access database

docker exec -it enms_postgres psql -U enms_user -d enms_db

# Check service health

docker compose ps

## Service URLs (Default Configuration)

| Service | URL | Default Credentials |
| --- | --- | --- |
| Main UI | http://localhost:8090 | demo / demo123 |
| Node-RED | http://localhost:1882 | (see .env) |
| Grafana | http://localhost:3000 | admin / (see .env) |
| PostgreSQL | localhost:5432 | enms_user / (see .env) |
| MQTT | localhost:1884 | (see .env) |

## Important Files

| Purpose | File Path |
| --- | --- |
| Environment Config | .env (create from .env.example) |
| Database Schema | backend/db_init/01_schema.sql |
| Main Dashboard | grafana/dashboards/fleet-operations.json |
| API Endpoints | python-api/app.py |
| Data Flows | node-red/flows.json |

Document Version: 1.0

Last Updated: 2025-12-09

Maintainer: ENMS Development Team